You are reading it correctly, but the key thing is:

ar <+/- ,32,16> [2] does not mean “A only has 2 total elements.”
It means the A input terminal accepts one vector slice of length 2 per handshake/cycle.

For your configuration:

M = 2
L = 2
N = 1

A is M x L = 2 x 2
B is L x N = 2 x 1
C is M x N = 2 x 1

and with:

Input pattern of A = Row Vector
Input pattern of B = Column Vector
Output pattern     = Row Vector

the node expects this:

| MATLAB object | LabVIEW terminal | What goes on the wire |
| --- | --- | --- |
| A = [3 4; 5 6] | ar [2] | one row of A at a time |
| B = [1; 2] | br [2] | one column of B |
| C = [11; 17] | cr [1] | one row of C at a time |

So for A, you do not wire [3 4 5 6] into ar.

You feed the two rows over time:

First accepted A vector:   ar = [3 4]
Second accepted A vector:  ar = [5 6]

That is why the constant created from the ar terminal is length 2. That is correct. The matrix has four total values, but the node is a streaming FPGA primitive, not a normal desktop 2D-matrix input. NI’s docs describe this as configuring the matrix size with M, L, and N, then choosing whether the function receives data by element, vector, row-wise, or column-wise. They also note that for a row-vector A(M x L) input, the input cycle count is M; for a column-vector B(L x N) input, the input cycle count is N.

For your exact MATLAB example:

A = [3 4;
     5 6];

B = [1;
     2];

C = A * B;

the LabVIEW feed should conceptually be:

A row 0: [3 4]
A row 1: [5 6]

B column 0: [1 2]

Then the result is:

C row 0: [11]
C row 1: [17]

Because your output pattern is Row Vector, cr is length [1]. That also makes sense: C is 2 x 1, so each row of C contains one value. You should see two valid output rows over time:

First valid output:   cr = [11]
Second valid output:  cr = [17]

If you want the output to appear as a single length-2 vector like MATLAB’s column vector, then change:

Output pattern = Column Vector

Then you would expect:

cr [2] = [11 17]

But your current row-vector output is not wrong. It is just streaming the 2 x 1 result one row at a time.

The coercion dot with a length-4 array is happening because ar is statically typed as a fixed-point vector of length 2. A length-4 array is the wrong terminal type. NI’s docs also point out that these FPGA linear algebra functions support scalar and vector fixed-point terminals, and that Matrix Multiply does not adapt the input pattern from the wire; the input pattern has to match the configuration dialog.

So the practical answer is:

Do not try to make ar [4].
Keep ar as [2].
Use a small counter/state machine to send [3 4] first, then [5 6].

Something like this conceptually:

row_index = 0  -> ar = [3 4]
row_index = 1  -> ar = [5 6]

and increment row_index only when the node actually accepts the A input, meaning your a valid is true and the node’s ready for a handshake indicates it is ready.

For this example, I would keep:

Input pattern of A = Row Vector
Input pattern of B = Column Vector

Do not switch to element-wise unless you specifically want to feed one scalar per clock. In element-wise mode you still would not feed [3 4 5 6] as a length-4 vector; you would feed scalar 3, then 4, then 5, then 6 over multiple cycles.

The mental model is:

Normal MATLAB:
  A is a full 2D matrix all at once.

LabVIEW FPGA Linear Algebra Matrix Multiply:
  A is streamed into the node as row vectors, column vectors, or scalar elements over time.

For your setup, ar [2] is exactly what I would expect. You are not missing two values; you need to provide the second row on the next accepted A input beat.